Simon Sanderson
MMCS Brightness Adjustment
15 April 2004


LCD Screen Brightness Adjustment 
for Mitsubishi Delica Royal Exceed 1996
It would appear that the brightness of the LCD display is controlled by the brightness as set for a conventional TV.  These instructions allow you to adjust the screen brightness and hence read Japanese even in direct sunlight!

Every so often my MMCS system as I shall call it from here (Mitsubishi Multi Communication System) goes mad and keeps displaying the screen shown in Section 11.   Until I can translate the Japanese I have no idea what it’s telling me.  When it starts doing this, the only way I can stop it is by starting the engine!

So for the sake of your own sanity, as soon as it does this to you (or before), just start the engine and follow the instructions below:

1. Centre Console and Controls

[image: image1.jpg]


This picture shows the centre console in my Delica.  I have labelled the buttons that we are interested in.  The bit labelled Touch-screen is what I am attempting to brighten up!

2. Turn on ignition or start engine

So the MMCS comes to life.  

3. Turn off side-lights & head-lights

When the lights are on the display is automatically dimmed (so that you don’t get dazzled at night by the internal cabin lighting etc).  Turning the lights off removes this annoyance from the equation.  That said however, I would recommend performing this ‘in the dark’ (i.e. not bright light), as the brightness changes will be more apparent.

4. Select TV Button

This puts the MMCS into TV mode.


5. Select Lower DISP Button

[image: image7.jpg]



When this is selected my MMCS shows a screen with loads of squares – these are the pre-programmed TV channels (Btw: none show a good picture but the sound is fine – I guess the Japanese TV system is different to Australia?). 


6. Select VTR Button (physically tap the screen as it is a touch-sensitive screen) 

This is the rectangle labelled VTR on the top-left of MMCS screen.  I am guessing that this switches to an optional Video Tape Recorder input, however I select it because it makes the ‘picture’ blank but steady and that makes the following steps easier to follow.

7. Touch/tap ‘Middle’ Button as shown

[image: image2.jpg]


[image: image3.jpg]Ky TUREET

1. Ry FUBLEBYVETDT
IITVEHITFEL

2. 1 5BLACETE
BEBNCOFFLETY




This now selects the picture adjustment screen.  On my system there appears to be 4 adjustments that can be made as mentioned previously.  Each adjustment mode, as below, shows a graphic of a slider-bar that slides from left to right increasing the adjusted setting as it goes.  


Pressing Mode Select (A) successively will ‘loop’ around the different settings:
Colour ( Contrast ( Brightness ( Hue ( Colour ( Contrast ( etc etc.

8. Select (touch-screen) Mode Select (A)

[image: image4.jpg]VTR @

Select this button for TV controls



Tap the Mode Select (A) several times until the correct option is displayed – this is when the graphic on the slider-bar Less (B) button looks like two Capital Bs   i.e BB

9. Increase the brightness

Now touch and hold the More (C) button.  The slider-bar will move to the right, bleeping as it goes, and the screen will brighten.  When the slider reaches the right then you are at maximum brightness.

If there’s no perceptible change, check that you have the right ‘mode’ or do the belt-n-braces approach and set all the sliders to the right/max!

10. Select another display

Wait for 5 or 10 seconds and the picture adjustment screen will automatically be exited.  Now select say the CD physical button.  The CD display should now appear.


You should see that the screen is brighter (assuming that it wasn’t already at the maximum brightness to start with).

The brightness setting etc will be retained when the ignition is off so you should only have to do this once thankfully.

11. Mad display

[image: image5.jpg]



As mentioned at the start this display pops up just to annoy me after about 5 minutes or so – the cure is to turn on the engine {{:-))


12. Play time

If you were unaware that the LCD screen was touch-sensitive I will hopefully have opened up a whole new driving experience for you!  This is because the Tape, CD changer unit, Air conditioning (front and back), TV, tour of Tokyo are all controlled in this manner.   Have fun playing with the new-fangled controls and learn some Japanese at the same time for free.

Remember keep your eyes on the road and hands on the wheel when driving though!

If anyone is interested I could cobble together a ‘user document’ following the above format as a user manual for the MMCS).  However this may take some time with experimenting on my part.  I would probably also need a Japanese girl-friend to do the aural work ;-) and experimenting with!

Simon Sanderson

Remove the Xs below to get my e-mail address (to stop spam web-crawlers)

orestone@X.optusnetX.com.au.X 

[image: image6.jpg]



My Baby



















