# User Tuneable Features

Intended Audience:	EcuTek tuners. Compatible ECUs:	US – AF423, UK-AF531

This new feature allows the car owner to subtly alter the configuration of an ECU supplied by an EcuTek dealer. Parameters such as rev limit, fuelling, ignition timing and boost pressure may be altered for situations such as racing on high octane fuels, adjusting boost control for unusual atmospheric conditions and dropping the rev limit before leaving the car with a valet. All of these features are available via DeltaDash. Furthermore, switching between pairs of boost maps and the configurable rev limit are available from the dashboard. To utilise this new feature, a special ROM file version must be programmed into the ECU, since the standard ECU does not contain this functionality.

# Selecting Available Features

The following user tuneable features may be enabled or disabled by setting or clearing bits in the ”user configuration data value”, accessible via Flash2002 in the “Data Value” maps section. Under the "OPTIONS" menu please enable the "User Level" setting to ADVANCED to see the "User Tune Config Byte".

| Bit | Hex Value | Function |
| --- | --- | --- |
| 0 | $01 | Idle Speed Adjustment |
| 1 | $02 | Rev Limit Adjust (Valet Mode) |
| 2 | $04 | Overall Ignition Adjustment |
| 3 | $08 | Ignition Learning Adjustment |
| 4 | $10 | Boost Pressure Adjustment |
| 5 | $20 | Duty Cycle Adjustment |
| 6 | $40 | Open Loop Fuelling Adjustment |
| 7 | $80 | Boost & Duty Map Switching |

Simply add the above values together for the features that should be enabled and enter the total in the user tune configuration data value from Flash2002. This allows only the features required to be enabled to achieve the required result. The features selected will then be accessible via DeltaDash.

An example of this would be that if you wanted to allow the User Tuner to have access to all features then enter FF in the “User Tune Config Byte” in “Data Values” in Flash 2002. If you wanted the features Overall Ignition Adjustment, Ignition Learning Adjustment, Boost Pressure Adjustment, and Duty Cycle Adjustment then add the hex numbers 04 + 08 + 10 + 20. This adds up to 3C.

# Feature Descriptions

## Idle Speed Control

Allows a wide idle speed rev range.

## Rev Limit Adjust

Allows adjustment of the rev limit used when in valet mode.

## Base Ignition Adjust

Allows up to 5 degrees of timing to be added or subtracted from the base ignition timing. This is useful when running race fuel, or tuning on the dyno.

## Ignition Learning Adjust

Scales the result of the ignition correction map lookup. Allows ECU to learn higher timing. This is useful for higher octane fuels. I.e. setting this value to 150% would give 50% more maximum ignition advance over and above the base map.

## Desired Boost Adjust

Allows up to 5 PSI of boost to be added or subtracted from the desired boost curve.

## Max Duty Adjust

Scales the maximum permissible wastegate duty. Useful for achieving higher boost specified with the desired boost adjust, or to calm down unstable boost in very cold weather.

## Open Loop Fuel Adjust

Adjusts the open loop air/fuel ratio. This scales the ‘percentage over stoich’ quantity. E.g. if the ECU is running 12.7:1, then that is two points over stoich. If this were scaled to 50%, then the ECU would run approximately 13.7:1.

## Boost & Duty Map Switching

Allows switching between pairs of boost and duty cycle maps. Boost and duty maps labelled ‘A’ to the user are the auto maps when edited from Flash2002. Maps labelled ‘B’ are the manual maps.

## Valet / Map Switch Mode

This is perhaps the most interesting feature to the average customer. This allows the driver to switch boost, duty and rev limit from the dashboard controls. By default, the ECU runs standard rev limit plus boost and duty maps ‘A’. Toggling valet mode switches to custom rev limit plus boost and duty maps ‘B’. Since the rev limit is configurable via DeltaDash, the customer may use this as a safety feature to prevent a second driver from using high revs and consequent vehicle speeds. Alternatively, the customer may leave the rev limit standard and the tuner may use the feature to provide two separate performance levels.

Holding full throttle and pressing the rear defog button toggles valet mode. This may be done when the engine is off or running, so long as the ignition is on at the time.

## Configuration Persistence

These new features have been configured to reset to neutral values whenever a standard ECU reset is requested via the diagnostics port from a select monitor or DeltaDash.

However, unlike other ‘programmed’ tuning, any of these parameters may be altered while the engine is running via DeltaDash, making adjustment live, quick and simple.