You need a 17mm spanner and 17mm socket, a large screwdriver to push the piston in and a 14mm socket. 

(Get impact sockets just incase some idiot has part rounded your bolts too!!)  

Take the wheel off 

Push piston back with large screw driver 

Loosen top 17mm caliper bolt with spanner 

Undo bottom 14mm bolt and swing the caliper up to remove pads 

Remove bottom 17mm bolt and swing whole caliper up

Replace disc 

Swing caliper back down and insert 17mm bottom bolt 

Put 2 wheel nuts on to hold disc steady 

Stick new pads in 

Replace 14mm bolt 

PUMP BRAKE PEDAL 

Remove retaining wheel nuts from disc 

Tighten top 17mm, bottom 17mm and 14mm bolts 

Replace wheel.

PUMP THAT BRAKE PEDAL AGAIN UNTIL SHES FIRM

Allow about 1 1/4 hours.