Wheels size and Fit
Ford Probe and Maxda 626 wheels fit.
The PCD you need is 5/ 114.
Make sure you fit the correct tyres. Specs should be on a label on drivers door 
http://myweb.tiscali.co.uk/mozzer/wheel%20fitment.htm
Use this site to calculate speedo error for tyre size 
http://www.miata.net/garage/tirecalc.html

use tyres with a load rating at least as high as those being replaced ( Estimas are very heavy).

http://www.alloyguide.co.uk/index.htm is link by Gerzenplaster - thanks. 

Full list of all vehicles and pcd's centre bore etc =
http://www.bikerlifestyle.co.uk/tech/pcd/pcd.htm by sussexsteve - thanks

The Wasp makes this interesting comment:-
Previa wheels will fit fine, but Estima wheels are too small for a previa, they go on ok but the car won't move because they jam against the brake disc.....how do I know this.......guess

Another good site: http://www.rochfordtyres.co.uk/search.asp?numrecordposition=1&strSize=15%22&strkeywords=5x114&strSort=Low&Make=Toyota&Model=Estima

A good point recently posted by Paulnjuls,
'note to anybody who has decided to fit large wheels to there bus,please check you have the correct size spiggots to centralize the wheels.If you dont they may move around and become loose as ours did today.For anybody choosing to fit RX8 wheels as we have you need a spiggot size of 60mm inside diameter and 67mm outside diameter.We have been told today that the bigger the diameter wheel you fit the more you rely on the spiggots,because you run low profile tyres they shock the wheel more hence they come loose'.

NO Probs Ian,

I hope you dont think Im treading on your toe's here  

This is the Sizes of the Lucies Wheels( Alloy's )

O.E Size: 15x6 e45 & 15x6.5 e50 (O.E Original Equipment )

When steping up a size of wheel from 15" to 16" you need to step down a profile 

On a Standard Wheel a tyre will be 205/65x15

On a 16" wheel a tyre will need to be 215/60x16

The P.C.D is 5 x 114.3  ( 5 stud with a 114.3mm diameter)

Ive listed an example of wheel size plus tyre size

Wheel                           Tyre
Size         Offset              Size

15x6.5    38    205/65x15  
15x7.0    35    205/65x15  
16x7.0    38    215/60x16  
16x7.5    37    215/60x16  
17x7.0    38    215/50x17  
17x7.5    37    215/50x17  
18x7.0    38    215/45x18  
18x7.5    37    215/45x18  
19x7.5    37    225/35x19


You also need to get a load rating.

Load                Load in          
Rating                 Kg ( Per wheel)             
89                     590- 
90                     600 
91                     615 
92                     630 
93                     650 
94                     670 
95                     690 
96                     710 
97                     730 
98                     750 
99                     775 
100                   800 
101                   825 
102                   850 
103                   875 
104                   900 
105                   925 
106                   950 
107                   975 

So a 3 tonne Bus needs at least a 750 kg tyre + 10%
so go for a 102 load rate 


A couple of good sites to add to Ians List for tyres and ahow to read them

http://www.chris-longhurst.com/carbibles/index.html?menu.html&tyre_bible.html

and

http://www.michelin.co.uk/uk/auto/auto_cons_bib_com_lir_pne.jsp

Mark