Rear Tail Light Bulb Replacement
First of all open the tail gate........... 
remove the 2 screws (see photo)


next slide the light cluster rearwards (may take a bit of force,some use a pair of rubber gloves to grip the cluster... )


to remove the bulb simply twist the holder from the cluster



when bulb is replaced simply slide cluster back into place & replace screws

H4 Head Light Bulb Replacement
working from behind the light unit...........
remove the wiring connector & black plastic collar


next remove the rubber seal........


remove the retaining clip which holds the bulb in place.......


remove bulb but make a note of the locating lugs (1,2&3) 
replace bulb & make sure it`s seated correctly in the locating lugs.......
in proper headlight alignment may result if bulb not seated correctly


re-fit retaining clip,rubber seal,plastic collar & wiring connector...........

Tailgate Bulb Replacement
Remove the 4 screws/retaining clips from the tailgate trim.........


gently prise the bottom edge of the trim off it`s retaining clips.......

remove trim to reveal bulbs.....(these might be behind a black cover but mines missing... )


simply twist the bulb holder from the tailgate cluster.......



once bulb changed re-fit back into cluster & refit all trims
be carefull when replacing trim as the clips can snap easily but can be bought at most car accessory shops

Front Fog/Aux/Driving Light Bulb Replacement



with the bonnet up... ..remove these 3 screws (might be cross-headed screws not bolts like mine)



now you can either remove the front lights completely or just leave them hanging on the wiring
to remove completely disconnect from the wiring connector.........


next remove the cover to expose the bulb....


remove the retaining clip holding the bulb in place & remove the bulb from the light unit...



disconnect the wire from the bulb & remove bulb

replacement is the reversal of the above but make sure the cut out`s on the bulb (small rectangle on one side &
semi-circle on the other) line up with the indentations in the light unit.......

Front Indicator Bulb Replacement


I have removed the front wheel just to make it easier to take photo`s.
i believe they can be changed without removing wheel........... 
You will have to remove the plastic wheel arch liner.............

the liner can then be twisted out of the way.........


if you look up inside the front wing you will see the rear of the bulb holder.......


as with the rear lights just twist the bulb out of it`s holder & replace.......
Re-fitting is the reversal of the above............. 

please excuse the dirt in the photo`s.........

Front Side/Turn Lights


first of all remove the fixing screw in the corner of the light.........


pull light unit out at an angle from it`s aperture..........


remove unit completely.........


twist off bulb holder from unit.......


remove bulb & replace......


re-fitting is a reversal of the above............... 

Note... the bulb is a 2 filament type similar to a stop/tail bulb..1 part for the side light &  the other
for the turn light(when headlights/indicators are on)