Top of Form

Changing the glow plugs Tutorial « on: February 26, 2007, 06:59:47 PM ». Here i do the glow plugs and I'm going to show you how i have done it, DISCLAIMERS NOTES:- IF PEOPLE WANT TO DO THE WORK, THEY DO SO AT THERE OWN RISK AND ME(JJ) NOR THE SITE TAKE NO RESPONSIBILTY FOR ANY OUT COME Here I'm going to show you how i did it, it took in total about 20Min's,and that is without taking the engine cover off, below i will show you in text and pictures, If you like easier access take the engine cover off :- http://www.estimaownersclub.org/community/index.php?topic=22977.0 First off you need to disconnected the battery as you will be playing with live wire so please do disconnected it, Also make sure there no dirt or fluid of any form sitting under the plugs or you could run in to problems, First off you need to take the plastic nut covers off each glow plugs with a straight edge screw driver:- Then remove the nuts on each glow plug :- And on number 4 glow plug (rear end of engine) there a thick wire on the you need to move that to one side :- Then there a metal strip you now can remove and put to one side And with a long reach socket you can undo the plugs and refitting is the reverse of taken them out but please make sure that the wire and metal strip is not touching anything only the plugs. Hope this helped Benefit :- better cold start, Cost :- about £30 for a set of 4 plugs, Tools :- 10mm socket, screw driver, 14mm long reach socket for glow plugs Hope you enjoyed the tutorial and have fun ....JJ..