Start by using a small flat bladed screwdriver to remove the covers on the grab handle. 

Unscrew the exposed screws and remove the handle. 
Gently pull away the door seal.

The door pillar trim can now be gently pulled away.

If you need to gain extra access you can remove the small piece of trim with the air vent, using a small scraper gently lever up along its length. (there are no screws, just clips)

And there you have it, refit in reverse order.

Thanks Estimamike for sharing your wisdom.