[image: image1.png]


[image: image2.png]


CHASSIS – G50 AND G55 MANUAL TRANSMISSIONS

[image: image3.png]


[image: image4.png]


CHASSIS- G50 AND G55 MANUAL TRANSMISSIONS


G50 AND G55 MANUAL TRANSMISSIONS
( DESCRIPTION

( The new Hilux has been adopted the G50 and G55 manual transmissions. The application for these manual transmissions are as followings:

	Engine Type
	Drive Type
	Transmission

	2KD-FTV
	2WD
	G50

	1TR-FE
	2WD
	G55


( A reverse synchromesh mechanism has been adopted for the reverse gear of these manual transmissions to enhance the ease of operation.
[image: image5.png]



(Specifications(
	Transmission Type
	G50
	G55

	Engine Type
	2KD-FTV
	1TR-FE

	Gear Ratio
	1st
	3.928
	4.452

	
	2nd
	2.333
	2.619

	
	3rd
	1.451
	1.517

	
	4th
	1.000
	(

	
	5th
	0.798
	0.854

	
	Reverse
	4.743
	4.472

	Oil Capacity     Liters (US qts, Imp.qts)
	2.2 (2.3, 1.9)
	(

	Oil Viscosity
	SAE 75W-90
	(

	Oil Grade
	API GL-4 or GL-5
	(

	Weight (Reference)*              kg (lb)
	41.3 (91.1)
	(


*: Weight shows the figure with the oil fully filled.
( Synchromesh Mechanism Specifications(
	Transmission Type
	G50
	G55

	Engine Type
	2KD-FTV
	1TR-FE

	1st
	Single Cone
	(

	2nd
	Single Cone
	(

	3rd
	Single Cone
	(

	4th
	Single Cone
	(

	5th
	Single Cone
	(

	Reverse
	Single Cone
	(


( REVERSE SYNCHROMESH MECHANISM

1. General

The reverse synchromesh mechanism consists of 2 shifting keys, shifting key spring, and shifting key return spring. 

[image: image6.png]





















2. Operation

( While shifting into 5th, the 2 shifting keys move in unison with the hub sleeve (F1). With "A" acting as the fulcrum, the 2 shifting keys move to push the synchronizer ring only for the amount of the lever ratio (F2). The shift effort can be reduced only for this amount.


( While shifting into reverse, with "B" acting as the fulcrum, the 2 shifting keys move to push the synchronizer ring (F2). As a result, a brake is applied to the rotation of the counter shaft and gear noise is prevented.






























































































































































































































































































































































































































































































































































Reverse Synchormesh


Mechanism





Shifting Key 


Return Spring








G50






































F2





233CH04





Synchronizer 


Ring





F1





B





Hub Sleeve Moving Direction





Shifting Key 





�





233CH03





Hub Sleeve





Shifting Key 





Shifting Key 





�





233CH01





Hub Sleeve





Shifting Key





Shifting Key 





�





233CH02





Synchronizer 


Ring





F1





F2





Hub Sleeve Moving Direction





A





Shifting Key 





�





233CH05





Shifting Key 





Shifting Key Spring





Snap Ring





Counter 5th Gear





Shifting Key Return Spring





Shifting Key 





Snap Ring





Shifting Key 





Shifting Key 





271CH04





�





Hub Sleeve





�





Hub Sleeve





Shifting Key Spring





Counter 5th Gear











