[image: image1.png]1

L

J]

i =
M)
%‘
|?

is)
=T

%

@

oL
L’EI
|

T

[

|
i
-‘mi-

WO

=
=1
LS
; ‘
S|
—3
——

llllllllll



[image: image2.png]B [ 1]

g
It

|
o\ |
QL

E i ISR |

[— 3
[T
||||IIIIIII|I|||
QE._
8]

o~
-

-
L
<

=

VAR

I
F——%
= ]

VTS




CHASSIS – R150, R150F, R151 AND R151F MANUAL TRANSMISSIONS

CHASSIS- R150, R150F, R151 AND R151F MANUAL TRANSMISSIONS


R150, R150F, R151 AND R151F MANUAL TRANSMISSIONS
( DESCRIPTION

( The new Hilux has been adopted the R150, R150F, R151, and R151F manual transmissions. The application for these manual transmissions are as followings:

	Engine Type
	Drive Type
	Transmission

	( 2TR-FE
( 1GR-FE
	2WD, 2WD PreRunner
	R150

	
	4WD
	R150F

	( 2KD-FTV

 High Version
( 1KD-FTV
	2WD, 2WD PreRunner
	R151

	
	4WD
	R151F


( A reverse synchromesh mechanism has been used for the reverse gear of these manual transmissions to enhance the ease of operation. The construction and operation of this mechanism are the same as the reverse synchromesh mechanism that has been adopted in the G50 series manual transmissions. For details, refer to G50 and G55 Manual Transmissions on page CH-xx.










(Specifications(
	Transmission Type
	R150
	R150F
	R151
	R151F

	Engine Type
	( 2TR-FE

( 1GR-FE
	(
	( 2KD-FTV

High Version

( 1KD-FTV
	(

	Gear Ratio
	1st
	3.830
	(
	4.313
	(

	
	2nd
	2.062
	(
	2.330
	(

	
	3rd
	1.436
	(
	1.436
	(

	
	4th
	1.000
	(
	1.000
	(

	
	5th
	0.838
	(
	0.838
	(

	
	Reverse
	4.220
	(
	4.220
	(

	Oil Capacity

Liters (US qts, Imp.qts)
	2.6 (2.7, 2.3)
	2.2 (2.3, 1.9)
	2.6 (2.7, 2.3)
	2.2 (2.3, 1.9)

	Oil Viscosity
	SAE 75W-90
	(
	(
	(

	Oil Grade
	API GL-4

or GL-5
	(
	(
	(

	Weight (Reference)*    kg (lb)
	50.8 (112)
	52.0 (114.6)
	50.8 (112)
	52.0 (114.6)


*: Weight shows the figure with the oil fully filled.
( Synchromesh Mechanism Specifications(
	Transmission Type
	R150, R150F
	R151, R151F

	Engine Type
	2TR-FE
	1GR-FE
	2KD-FTV

High Version
	1KD-FTV

	1st
	Single Cone
	Triple Cone
	(
	(

	2nd
	Triple Cone
	(
	(
	(

	3rd
	Single Cone
	(
	(
	(

	4th
	Single Cone
	(
	(
	(

	5th
	Single Cone
	(
	(
	(

	Reverse
	Single Cone
	(
	(
	(






























































































































































































































































































































































































































































































































































Reverse Synchormesh


Mechanism











R150









































































































































R150F





233CH09








271CH02





�








�











Reverse Synchormesh


Mechanism











