BO-12
[image: image1.png]


BODY ― BODY STRUCTURE

BO-11
[image: image2.png]


BODY ― BODY STRUCTURE

	BODY


BODY STRUCTURE

( LIGHTWEIGHT AND HIGHLY RIGID BODY

1. High Strength Sheet Steel
In the new Hilux, high-strength steel is used as following illustration in order to realize excellent body rigidity and lightweight body.
[image: image3.png]QOGS
QOG0

e e s tatatets o Bt
2 atatetetels:

RRRTR
e e T R KRR
i S RN
R I R KRR

R W Lo
oo e, B
R IR T TR
o] B R SRR
R R Rl KX
3R RS T R
/] 5 S RO
- \ OGO QOO0
R R
% RIS
£ DOO QOO
ey bR
5% %
A i R
Lol b, Yooisisisinimstones, B
. B R RO R
o O O 3
SRR 3 o0
O00000DTN0
ORI e

.......
s QOOOOOOOGEUSOOS
A ABRENIOOOTEBRLS

QOO
R ]

e arararetaress
QOB
O OB
s 200000
O DEINCTIHS

. Al
Q0 Q0
T, PRSI
QR 9 D
T R
R %
e
% — e

| ﬁ.@‘\i? /

[

e Aunwmmmmmmmmuuuuﬁ
O
KRR
SRR 5
R R s %%

[——
w~
A
I 000,





　 

( SAFETY FEATURES

1. General

The impact absorbing structure of the new Hilux can help absorb the energy of impact in the event of a front or side collision. This structure also realizes high-performance occupant protection through the use of reinforcements that help minimize cabin deformation.

2. Impact Absorbing Structure for Frontal Collision

Frame
The brace has been added to the lower front portion of the frame. 
In the event of a frontal collision, the impact energy is transmitted to the No.1 cross member through the brace. Thus, the moment of the backward rotation at the No.1 cross member occurs in combination with the moment of the forward rotation at the front suspension tower. This helps minimize the floor deformation caused by the front suspension tower collapsing.
[image: image4.png]ol L L LLLLLLA
]

””””“ N
\\

R

'!l’ﬁ\)g

.





[image: image5.png]oo Yere e ST, AOOOOAI LU
e et atatata gty iyt gy g g a b e T e e e T TR T OO OO OO e OO OO OO OO OO RO
O R S S S o R S
31O OGN atatatetetetetetetetetetete s
2 S atatetets





[image: image6.png]‘%47’(’/‘) ~
,»‘,Iv’l‘ ~
'/ .‘f‘;*?\?
A e
2D
T LA

" Z
K 2

\(:e‘





[image: image7.png]



[image: image8.png]Sa

S0 o

TR

",
Qldo
fatatetele
Teratatetetels,
fasatetatetetetetetels
Fasatatatatatatetetetetetetste
Baeratatatatatatatatatatata ettt

o
SOO00000
2atetetetetels!

ity
S
3

¥





[image: image9.png]SORRIR
et

SARIOOO0S
et
BB
o

)

S R COOOANARIOOODS,
b0

POOOOCENINIY




[image: image10.png]SOOOO0000
CRNHRHNNNN

atetels!

R
e

&
5

ettt e T
&

ROCOOAASS

.
.
.
.
.
.
.
.
.
..

SRR
e
SRR AR,

¢
&
&
&
&
&
&
&
&
&
&
&





Cabin
The optimally allocated panels and reinforcements allow the front impact energy received from the front end and frame to be dissipated and absorbed by the front pillar, rocker, and floor reinforcement, in order to minimize the deformation of the cabin.

[image: image11.png]O ooO0

Ooo O

T T

00

QOOO000
L
atatetetetetetetetety]
atatatatatetetetetets!
atatatatatetetetete %]
seeserese e etens)
atetetsts!

OO0





3. Impact Absorbing Structure for Side Collision
( Impact energy of a side collision directed to the cabin area is dispersed throughout the body via pillar reinforcements, side impact protection beam, and floor cross member. 

( This dispersion of energy helps keep the energy directed to the cabin to minimum level. 

[image: image12.png]&
¢
Aoatatetetels!
SOOOHOHOOS

Qo
O SO
D000 2 3
B I R ARK
B AR AR AR EBER s
B RBBEEE





( A Head Impact Protection Structure is used. With this type of construction, if the occupant’s head hits against the roof side rail and pillar in reaction to a collision, the inner ribs of the roof side rail and pillar collapses to help reduce the impact.

[image: image13.png]AN ———

o|O ol o

—

0

0
=0
©
=0

Al

’ i
.
o W (@] (@)
g 11 S— A o —
0 0 D¢ 0
p 4
0 0
oy o
e © =
YR \|A_o Aamimat 9 vV
Wu@hu e /5 b o000 00 0 [do0 C=rrt
n 5 . ..
<
0 Mwm O Hlo
Ho
O
o] H o]
o] 'lo
02 B i
H il | i
ou o F\ Ho
N o | | N
0 |= N L oﬁ.lz 0
ol n° = = i Lo
L o || = o | |
H Ho o = = - o Ho
n14 @ ullls Ns—=/] 1o @ ‘\n
1 il —— W [e] 5 1
Ho@ 1 _JJr._._|L__ I __I I__I__i I __rn_._|L_L._ @o
= o @ E 7 7 [©olo @ 0 )
[ | _____ | P_ [T T T
00 < m
o
[o]
N\
O o O
OO
0
(o]
o] [e]





[image: image14.png]



[image: image15.png]



( RUST-RESISTANT BODY

1. General

Rust-resistant performance is enhanced by extensive use of anti-corrosion sheet, as well as by an anti-corrosion treatment, which includes the application of anti-rust wax, sealer and anti-chipping paint to easily corroded parts such as the hood, doors, and hinges.

2. Anti-Corrosion Sheet Steel 

Anti-corrosion sheet steel is used as the following illustration.


3. Wax and Sealer

Wax and sealer are applied to the hemmed portions of the hood, door panels, fuel lid, door hinge and tailgate hinge to improve rust-resistant performance.

4. Under Coat

A PVC (Polyvinyl Chloride) coating is applied to the front wheel housings of the cab.

5. Anti-Chipping Application

On Australian models, PVC is applied to the rocker panel area, fenders, and below the deck side panels to enhance anti-chipping performance and ensure rust resistance.

( LOW VIBRATION AND LOW NOISE BODY

1. General

( The cab mount area has adopted a collar construction to improve the rigidity of its force application point and minimize the transmission of vibration to the body.
( An effective application of vibration damping and noise suppressant materials reduces engine and road noise.

2. Cab Mount 

On the new Hilux, the area to which the cab is mounted has adopted a collar construction to improve rigidity and minimize the transmission of vibration to the body.


3. Sound Absorbing and Vibration Damping Materials

( Asphalt sheet is used to reduce engine and road noise.
( Adoption of a dash panel silencer and hood silencer have reduced engine and road noise and improves the quietness in the compartment.


( Foamed urethane sponge and foamed seal material are applied on to the roof panel and pillars to reduce wind and road noise.




( Reinforcements have been optimally allocated to the back panel area of the single cab and double cab models to minimize droning noise. On all models, a stamped shape has been added to the deck panel on the cabin side to prevent it from resonating with the back panel and minimize droning noise.








4. Reducing Wind Noise
The clearances between the door sash and the front pillar and the roof have been reduced to reduce wind noise.








Double Cab Model





Double Cab Model





: Head Impact


Protection


Structure





D





C





�








271BO10





A





B – B Cross Section


(Except Single Cab Model)





Hood Silencer





Dash Panel Inner Silencer





D





271BO03





Side Impact 


Energy





�








F





F





E





E





D





Reinforcement








D





C





C





B





B





Double Cab Model





A





A





F - F Cross Section





D - D Cross Section





E - E Cross Section





C – C Cross Section





�














Reinforcement








Reinforcement








B





No.2 Cab Mount








Double Cab with LHD Model





C – C


Cross Section





�











Air　Flow





�








Air　Flow





B





C





Double Cab Model





Front Impact 


Energy





B – B


Cross Section





Deck Panel








Floor Panel





�

















271BO08





A





Single Cab Model





A – A


Cross Section





271BO11





: Sponge





Double Cab Model





: Asphalt Sheet





Double Cab Model





: Anti-Corrosion


 Sheet Steel





271BO02





Double Cab Model








�








D





271BO12





C





271BO06





A - A Cross Section





271BO07








A





�


               





D





C





B





: Foamed Seal Material


 (Only for Double Cab Model)








A





A-A Cross Section





Extra Cab Model





D – D


Cross Section





B





A








A








Single Cab Model








�








Front








271BO09





: Foamed Seal Material





: High-strength Sheet Steel





�








B – B


Cross Section











Single Cab Model





Extra Cab Model








271BO01





Brace





B





Upper View





B





B-B Cross Section





: PVC Coating Area








Double Cab Model





Only for Double Cab Model 





Frame








Body Shell








No.3 Cab Mount


(For Double Cab)








No.3 Cab Mount


(For Extra Cab)








Collar








D-D Cross Section








C-C Cross Section








No.3 Cab Mount


(For Single Cab)








B-B Cross Section








A-A Cross Section








A





Dash Panel Outer Silencer


(Only for Diesel Engine Model) 





�








271BO05





: PVC Coating Area








�








: Edge Seal





�














�











271BO04





Front Impact 


Energy





: Energy  Absorbing


Rib





271BO13





Double Cab Model








�








271BO15





271BO14





Extra Cab Models





Single Cab Models








