BE-4
[image: image1.png]i



BODY ELECTRICAL –LIGHTING

BE-3
[image: image2.png]


 BODY ELECTRICAL –LIGHTING


	BODY ELECTRICAL


LIGHTING 

( HEADLIGHT

( The multi-reflector type headlights are used as standard equipment.

( With the multi-reflector type headlights, the light from the bulbs is dispersed and distributed through multiple parabolic shaped reflectors. As a result, the lens cut pattern is no longer provided in the center of the lens, thus realizing a clear look.

[image: image3.png]Y

Y





(Light Distribution Imaginary Diagram(
[image: image4.png]A

Y





[image: image5.png]










( LIGHT REMINDER SYSTEM
( This system is used as standard equipment on the SR grade model.

( This system sounds the buzzer in the combination meter to warn the driver that the lights remain ON if the driver's door is opened while the headlights or taillights are ON when the ignition key is turned from the ON to ACC or LOCK positions. The buzzer is located inside the combination meter.

( This system is controlled by the meter ECU.

(System Diagram(

( ILLUMINATED ENTRY SYSTEM

( All Thai, Malaysian and Philippine SR grade models, as well as all Australian models, except 2TR-FE engine models, are equipped with the illuminated entry system.
( This system controls the key illumination in accordance with the opening and closing of the doors.
( Two types of body ECU have been provided, depending on the vehicle specifications. Key illumination control differs according to the body ECU type. For details, refer to the table below.

	Wireless Door Lock System
	Theft Deterrent System
	Outline

	○
	(
	( Illuminates when the any door is opened.
( Turns OFF when the ignition switch is turned from OFF to ON.
( Turns OFF when the all doors are locked.
( Turns OFF the key illumination 7.5 seconds after the all doors is closed
Key illumination control of KUN26R-PRPDYQ operates as described above, regardless of the setting.

	○
	○
	( Illuminates when the driver's door is opened.
( Turns OFF the key illumination 5 seconds after the driver's door is closed

	(
	(
	


(System Diagram(



Layout of Main Components

















































































































































































































































































































































































































































































































































































271BE64





Buzzer





Wireless Door Lock Receiver*1





Courtesy Switch


(Driver Door)





Meter ECU





Ignition Switch





Light Control 


Switch











PCTY




















271BE05





DCTY








Passenger Door


Courtesy Switch


(Front and Rear*2)








Rear Door Courtesy Switches*2





Front Door


Courtesy Switches





Ignition Switch


( Key Illumination





271BE02





Multi-Reflector Type Headlight





Light Distribution





Lens





Headlight Bulb





Reflector


(Multi Parabolic Shape)





�








�











Driver Door


Courtesy Switch











Key Illumination





Integration Relay


( Body ECU





ILE








Ignition Switch





�











Double Cab LHD Model











*1: Model with Wireless Door Lock Remote Control System


*2: Only for Double Cab Model





271BE03





*1: Model with Wireless Door Lock Remote Control System


*2: Only for Double Cab Model








RDA








�








Wireless Door Control Receiver*1





Integration Relay


( Body ECU





271BE01





Combination Meter





Multi-reflector





�











