[image: image1.png]PASSENGER

e e e e e e e e e



[image: image2.png]__________________________________




BODY ELECTRICAL – SEAT BELT REMINDER SYSTEM
[image: image3.png]vl o/




BODY ELECTRICAL – SEAT BELT REMINDER SYSTEM

SEAT BELT REMINDER SYSTEM

( DESCRIPTION

( A seat belt reminder system is used. If the seat belt is not buckled, the reminder light flashes and the buzzer sounds. 

( When the ignition switch is turned ON, this system determines whether or not the seat belt is buckled depending on whether the signal from the switch in the seat belt buckle is ON or OFF. 
( The occupant detection sensor provided in the seat cushion of the front passenger seat determines whether or not an occupant is seated in the front passenger seat.

( On the Australian model, the buzzer sounds when the vehicle speed exceeds 15 km/h (9.3 mph) to alert the driver that the driver seat belt is unbuckled. After 30 seconds, the seat reminder system emphasizes the alert by changing the cycle of the buzzer for 90 seconds.
( The specifications of this system vary by destination as indicated in the table below.

	Item
	Thailand
	Australia
	Philippines
	General

Countries
	Central and South America
	Malaysia


	Seat Belt Reminder Light
	For Driver Seat 
	Blink
	Blink
	Blink
	Blink*1
	Blink*3
	Blink


	
	For Front Passenger Seat
	(
	Blink
	(
	Blink*2
	Blink*3
	(


	Buzzer for Driver Seat
	(
	Sound
	(
	(
	(
	(



*1: Only for Models with Driver Airbag 

*2: Only for Models with Front Passenger Airbag 

*3: Only for SR grade Model

(System Diagram(






( OCCUPANT DETECTION SENSOR

The occupant detection sensor, which is enclosed in the seat cushion of the front passenger seat, is used to detect whether or not the front passenger seat is occupied.

This sensor, which is shaped as illustrated below, consists of a construction in which two sheets of electrodes sandwich a spacer. When the occupant is seated, the electrode sheets come in contact with each other through the hole that is provided in the spacer portion, thus enabling the current to flow.

Thus, the sensor detects whether or not an occupant is seated in the front passenger seat.















































































































































































































































































































































































































































































































































































































































Spacer








Center Cluster





271BE57





Occupant Detection


Sensor











271BE62











Upside View





Combination Meter





Seat Belt 


Reminder Light


(Driver)





Seat Belt 


Reminder Light


(Front Passenger)





For Front Passenger Seat





Speed Sensor





Seat Belt Buckle 


Switch (Front Passenger)





271BE58





Seat Belt Buckle 


Switch (Driver)





For Driver Seat





�














�














Front Passenger Seat





Sensor ON





Occupant





Occupant Detection Sensor





Sensor OFF





Electrode Sheet





Electrode Sheet








�








Meter ECU








